Railway Station Passenger Flow

Submission 1 – High-Level Problem and Data Set

EEI6373 – Performance Modelling

Akila Dhambure Liyanage  |  320267120

# 1. Identification of the Complex System

This mini project studies the passenger flow through the ticket concourse of a major railway station during the morning peak period (07:00 – 09:00). The station is modelled as a complex service system in which passengers must pass through a chain of stages: ticket acquisition, entry gates, platform access and boarding. The system is complex because the stages are coupled, arrivals are stochastic, demand is sharply concentrated in a short window, and the performance of one stage directly constrains the stages that follow.

Two passenger classes are distinguished. Normal ticket passengers must queue at one of three ticket counters before proceeding to the gates, whereas Online ticket passengers bypass the counters entirely and go straight to the gates. This mixed demand pattern creates an uneven, time-varying load on the counter subsystem and makes the ticket counters the natural bottleneck candidate.

## 1.1 Why the System Is Complex

Stochastic behaviour – passenger arrivals are random, and service times vary passenger by passenger.

Coupled stages – congestion at the counters propagates into gate and boarding queues.

Class-dependent demand – Normal and Online passengers place very different loads on the counters.

Peak concentration – almost all demand occurs within two hours, producing a heavy transient.

Limited resources – fixed counter capacity and gate capacity must be dimensioned to absorb the peak.

# 2. Performance Objectives

The project defines the following quantifiable performance objectives for the system, consistent with queueing-theoretic measures:

O1 – Identify the bottleneck stage of the passenger flow and quantify its utilisation under normal and busy demand.

O2 – Estimate the expected waiting time at the ticket counters (Wq), the probability that a passenger has to wait (P(wait)), and the expected queue length (Lq) under the baseline peak demand.

O3 – Determine whether the current configuration (c = 3 counters, μ = 40 passengers/hour) is stable, and find the arrival-rate capacity limit.

O4 – Evaluate what-if interventions (additional counters, faster service) and show how they reduce waiting under high load.

O5 – Validate the analytical model (M/M/c, Erlang-C, Little's Law) against a passenger-level simulation to confirm the estimates are trustworthy.

# 3. Data Set

The data set is a passenger-level record of the 07:00 – 09:00 peak period generated by a FIFO M/M/c discrete-event simulation (c = 3, μ = 40/hour). It contains 400 passengers: 180 Normal (45.0%) and 220 Online (55.0%), corresponding to arrival rates of λ = 90/hour and λ = 110/hour respectively. The file is stored in the repository at data/passenger_flow_peak_hours.csv.

Important: the passenger-level data are simulated, not real station observations. They are produced so that the data set validates the analytical model used in the project.

## 3.1 Columns

| Column | Description |
| --- | --- |
| Passenger_ID | Unique passenger identifier (P001 … P400) |
| Arrival_Time | Time of arrival into the concourse (07:00:00 – 09:00:00) |
| Ticket_Type | Passenger class: Normal (uses counters) or Online (bypasses counters) |
| Counter_Wait_Min | Waiting time at the ticket counters (min); 0 for Online passengers |
| Counter_Service_Min | Service time at the ticket counters (min); 0 for Online passengers |
| Gate_Wait_Min | Waiting time at the entry gates (min) |
| Gate_Service_Min | Service time at the entry gates (min) |
| Boarding_Wait_Min | Waiting time on the platform before boarding (min) |
| Total_Time_Min | Total time in system = sum of the five component times (min) |

## 3.2 Sample Rows

| ID | Arrival | Type | C-Wait | C-Serv | G-Wait | G-Serv | B-Wait | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P001 | 07:00:02 | Online | 0.00 | 0.00 | 0.65 | 1.47 | 8.53 | 10.65 |
| P002 | 07:00:04 | Normal | 0.00 | 0.88 | 0.19 | 0.76 | 8.88 | 10.71 |
| P003 | 07:00:12 | Online | 0.00 | 0.00 | 0.33 | 0.41 | 1.00 | 1.74 |
| P004 | 07:00:25 | Online | 0.00 | 0.00 | 0.93 | 0.71 | 1.56 | 3.19 |
| P005 | 07:00:46 | Normal | 0.00 | 0.36 | 0.05 | 0.85 | 1.40 | 2.67 |

## 3.3 Summary Statistics

All 400 passengers (times in minutes):

| Measure | Counter Wait | Counter Service | Gate Wait | Gate Service | Boarding Wait | Total Time |
| --- | --- | --- | --- | --- | --- | --- |
| Mean | 0.55 | 0.66 | 0.49 | 0.40 | 5.05 | 7.14 |
| Std dev | 1.70 | 1.27 | 0.51 | 0.40 | 4.72 | 5.36 |
| Min | 0.00 | 0.00 | 0.05 | 0.05 | 1.00 | 1.10 |
| Median | 0.00 | 0.00 | 0.33 | 0.29 | 3.52 | 5.77 |
| Max | 8.49 | 13.94 | 3.14 | 3.10 | 29.81 | 31.80 |

Normal passengers only (the class that uses the counters; n = 180):

| Measure | Counter Wait | Counter Service | Total Time |
| --- | --- | --- | --- |
| Mean | 1.22 | 1.46 | 8.67 |
| Std dev | 2.36 | 1.56 | 5.61 |
| Median | 0.00 | 1.08 | 7.26 |
| Max | 8.49 | 13.94 | 31.80 |

The simulated mean counter service time of 1.46 min corresponds to a service rate of about μ = 41.2 passengers/hour (target 40), and the simulated mean counter wait of 1.22 min is close to the analytical M/M/c value of Wq = 1.14 min, which confirms that the data set is consistent with the model.

# 4. Repository and Next Steps

The complete project (data set, scripts, figures and this document) is stored in a git repository: https://github.com/dhambure98/PM-MP-Railway-Station.git.

In the next phase the analytical M/M/c (Erlang-C) model will be applied to these data to compute utilisation, waiting times and queue lengths, followed by a what-if capacity analysis and the final report.